# Packet Tracer - Skills Integration Challenge

| 1.3.1.3 Packet Tracer - Skills Integration Challenge.pka | Connect Console cable from User-01 (RS 232) to ASw-1 (Console) ASw-1 -> Startup Config Switch>enable Switch(config)#copy running-config startup-config ENTER ASw-1 -> Banner MOTD Switch(config)#banner motd "Warning, you must be admin" ASw-1 -> Console Line -> Password Switch(config)#line console 0 Switch(config-line)#password xAw6k Switch(config-line)#exit ASw-1 -> Enable Secret Switch(config)#enable secret 6EBUp ASw-1 -> Host Name Switch(config)#hostname ASw-1 ASw-1 -> Ports -> Vlan1 -> IP Address/Subnet Mask Switch#configure terminal ASw-1(config)#interface vlan 1 ASw-1(config-if)#ip address 10.10.10.100 255.255.255.0 ASw-1 -> Ports -> Vlan1 -> Port Status Switch(config-if)#no shutdown Switch(config-if)#exit ASw-1 -> Service Password Encryption ASw-1(config)#service password-encryption ASw-1 -> VTY Lines -> VTY Line 0 -> Password ASw-1(config)#line vty 0 ASw-1(config-line)#password xAw6k ASw-1(config-line)#exit Connect Console cable from User-02 (RS 232) to ASw-2 (Console) Do all ASw-1 steps (except hostname = ASw-2 and Vlan1 = ip address 10.10.10.150 255.255.255.0) Open “IP Configuration” for User-01 IP Address: 10.10.10.4 Subnet Mask: 255.255.255.0 Open “IP Configuration” for User-02 IP Address: 10.10.10.5 Subnet Mask: 255.255.255.0 |
| --- | --- |
| 2.3.1.2 Packet Tracer - Skills Integration Challenge.pka | Click Switch -> CLI S1 -> Banner MOTD Switch>enable Switch#configure terminal Switch(config)#banner motd "Warning, you must be admin" S1 -> Console Line -> Password & Login Switch(config)#line console 0 Switch(config-line)#password cisco Switch(config-line)#login Switch(config-line)#exit S1 -> Enable Secret Switch(config)#enable secret class S1 -> Host Name Switch(config)#hostname S1 S1 -> IP Domain Name S1(config)#ip domain-name cisco.com S1 -> Ports -> FastEthernet0/x -> Port Status S1(config)#interface range FastEthernet0/1-24 S1(config-if-range)#shutdown S1(config-if-range)#exit S1 -> Ports -> FastEthernet0/1 & 2 -> Port Security -> Enabled & Port Security Violation S1(config-if-range)#interface range FastEthernet0/1-2 S1(config-if-range)#switchport mode access S1(config-if-range)#switchport port-security S1 -> Ports -> FastEthernet0/1 &2 -> Port Security -> Maximum Static MACs (config-if-range)#switchport port-security maximum 2 S1 -> Ports -> FastEthernet0/1 & 2 -> Port Security -> Sticky Enabled S1(config-if-range)#switchport port-security mac-address sticky S1(config-if-range)#exit S1 -> Ports -> GigabitEthernet0/1 & 2 -> Port Status S1(config)#interface range GigabitEthernet0/1-2 S1(config-if-range)#shutdown S1(config-if-range)#exit S1 -> Ports -> Vlan 1 -> IP Address & Subnet Mask S1(config)#interface vlan 1 S1(config-if)#ip address 10.10.10.2 255.255.255.0 S1 -> Ports -> Vlan 1 -> Port Status S1(config-if)#no shutdown S1(config-if)#exit S1 -> Service Password Encryption (config)# service password-encryption --- Generate RSA 768 bits token S1(config)#crypto key generate rsa How many bits in the modulus [512]: 768 % Generating 768 bit RSA keys, keys will be non-exportable...[OK] --- S1 -> SSH Server -> SSH Version S1(config)#ip ssh version 2 S1 -> Usernames -> Username (hit “secret password”) S1(config)#username admin secret ccna S1 -> VTY Lines -> VTY Line 0 -> Login S1(config)#line vty 0 S1(config-line)#login local S1 -> VTY Lines -> VTY Line 0 -> Transport Input S1(config-line)#transport input ssh S1(config-line)#exit Restart only two ports S1(config)#interface range FastEthernet0/1-2 S1(config-if-range)#no shutdown Open PC1 and PC2 -> Desktop -> IP Configuration Set IP Address: 10.10.10.10 or 10.10.10.11 Subnet Mask: 255.255.255.0 Gateway: 10.10.10.1 Open Command Prompt Ping: 10.10.10.2 (Switch) Ping 10.10.10.10 (PC1) Ping 10.10.10.11 (PC2) |
| 2.5.1.2_Packet_Tracer_-_Configure_Cisco_Routers for_Syslog,_NTP,_and_SSH_Operations[1].pka | R1 -> Logging -> Service timestamp log R1>enable Password: (ciscoenpa55) R1#configure terminal R1(config)#service timestamps log datetime msec (Check on the Dashboard IP Address of “NTP Server”) R1 -> NTP Client -> NTP Server Information -> IP R1(config)#ntp server 192.168.1.5 R1 -> NTP Client -> NTP Server Information -> Update Calendar R1(config)#ntp update-calendar (Verify: R1(config)#do show ntp status) (Verify: R1(config)#do show clock) R1 -> SYSLOG Client -> Server Addresses R1(config)#logging host 192.168.1.6 (Verify: R1(config)#do show logging) Click on “Syslog Server” -> Services -> SYSLOG Type in R1 -> CLI : R1(config)#exit Do the same steps in R2 and R3 routers --- R3 -> IP Domain Name R3(config)#ip domain-name ccnasecurity.com --- Generate RSA 1024 bits token (see notes) S1(config)#crypto key generate rsa How many bits in the modulus [512]: 1024 % Generating 768 bit RSA keys, keys will be non-exportable...[OK] Verify: R3(config)#do show ip ssh) --- R3 -> SSH Server -> SSH Authentication Retries R3(config)#ip ssh authentication-retries 2 R3 -> SSH Server -> SSH Timeout R3(config)#ip ssh time-out 90 R3 -> SSH Server -> SSH Version R3(config)#ip ssh version 2 R3 -> User Names -> Username (Verify: R3(config)#username SSHadmin privilege ? <0-15> User privilege level) R3(config)#username SSHadmin privilege 15 secret ciscosshpa55 R3 -> VTY Lines -> VTY Line 0 to 4 -> Login R3(config)#line vty 0 4 R3(config-line)#login local R3 -> VTY Lines -> VTY Line 0 to 4 -> Transport Input R3(config-line)#transport input ssh R3(config-line)#exit Open PC-C -> Desktop -> Command Prompt C:\>telnet 192.168.3.1 Trying 192.168.3.1 ...Open [Connection to 192.168.3.1 closed by foreign host] C:\>ssh -l SSHadmin 192.168.3.1 Password: (ciscosshpa55) R3#exit |
| 3.3.3.3_Packet_Tracer_-_Explore_a_Network[1].pka | It’s simulator demo from “Sales” PC |
| 3.4.1.2 Skills Integration Challenge.pka | Only S3 CLI is available we use connect console cables. Open S3 CLI (some conflict Gig0/2 connection, temporary shutdown) S3>enable S3#configure terminal S3(config)# S3(config)#interface GigabitEthernet0/2 S3(config-if)#shutdown S3(config-if)#exit S3 -> Banner MOTD S3(config)#banner motd "Authorized Access Only!!" S3 -> Console Line -> Password S3(config)#line console 0 S3(config-line)#password letmein S3(config-line)#exit S3 -> Default Gateway S3(config)# ip default-gateway 172.31.88.1 S3 -> Enable Secret S3(config)#enable secret itsasecret S3 -> Ports -> FastEthernet0/11 -> Access VLAN S3(config)#interface range FastEthernet0/7-12 S3(config-if-range)#switchport access vlan 10 S3(config-if-range)#exit S3 -> Ports -> FastEthernet0/18 -> Access VLAN S3(config)#interface range FastEthernet0/13-20 S3(config-if-range)#switchport access vlan 20 S3(config-if-range)#exit S3 -> Ports -> FastEthernet0/6 -> Access VLAN S3(config)#interface range FastEthernet0/1-6 S3(config-if-range)#switchport access vlan 30 S3 -> Ports -> FastEthernet0/6 -> Port Security -> Enabled S3(config-if-range)#switchport mode access S3(config-if-range)#switchport port-security S3 -> Ports -> FastEthernet0/6 -> Port Security -> Maximum Static MACs (config-if-range)#switchport port-security maximum 2 S3 -> Ports -> FastEthernet0/6 -> Port Security -> Port Security Violation S3(config-if-range)#switchport port-security violation restrict S3 -> Ports -> FastEthernet0/6 -> Port Security -> Sticky Enabled S3(config-if-range)#switchport port-security mac-address sticky S3(config-if-range)#exit S3 -> Ports -> FastEthernet0/4 & 0/14 & 0/24 -> Port Status S3(config)#interface range FastEthernet0/1-24 S3(config-if-range)#shutdown S3(config-if-range)#exit Check Dashboard - enable F0/6 & F0/11 & F0/18 S3(config)#interface FastEthernet0/6 S3(config-if)#no shutdown S3(config-if)#exit … S3 -> Ports -> GigabitEthernet0/2 -> Native VLAN S3(config)#interface GigabitEthernet0/2 S3(config-if)#switchport trunk native vlan 99 S3 -> Ports -> GigabitEthernet0/2 -> Nonegotiate S3(config-if)# switchport nonegotiate S3 -> Ports -> GigabitEthernet0/2 -> Port Mode S3(config-if)#switchport mode trunk S3 -> Ports -> GigabitEthernet0/2 -> Trunk VLANs -> 10 & 20 & 30 & 88 & 99 S3(config-if)#switchport trunk allowed vlan 10 etc. or S3(config-if)#switchport trunk allowed vlan all S3(config-if)#exit S3 -> Ports -> Vlan88 -> Port Status S3(config)#interface vlan 88 S3(config-if)#no shutdown S3 -> Ports -> Vlan88 -> IP Address / Subnet Mask S3(config-if)#ip address 172.31.88.4 255.255.255.0 S3(config-if)#exit S3 -> Service Password Encryption (config)# service password-encryption S3 -> VLANS -> VLAN 10 -> VLAN Name S3(config)#vlan 10 S3(config-vlan)#name Sales S3(config-vlan)#exit (Assign name for other VLANs) S3 -> VTY Lines -> VTY Line 0 -> Password S3(config)#line vty 0 S3(config-line)#password c1$c0 S3(config-line)#exit Restart GigabitEthernet0/2 S3(config)#interface GigabitEthernet0/2 S3(config-if)#no shutdown |
| 3.6.1.2_Packet_Tracer_-_Configure_AAA_Authentication_on_Cisco_Routers[1].pka | R1 -> AAA -> New-model R1>enable Password: (ciscoenpa55) R1#configure terminal R1(config)#aaa new-model R1 -> AAA -> Authentication -> Authen Command 1 R1(config)#aaa authentication login TELNET-LOGIN local R1 -> AAA -> Authentication -> Authen Command 2 R1(config)#aaa authentication login default local R1 -> Console Line -> AAA Method List Name R1(config-line)#line console 0 R1(config-line)#login authentication default R1(config-line)#exit R1 -> User Names -> Username username Admin1 secret admin1pa55 R1 -> VTY Lines -> VTY Line 0 to 4 -> AAA Method List Name R1(config)#line vty 0 4 R1(config-line)#login authentication TELNET-LOGIN R1(config-line)#exit Router R2 R2 -> AAA -> New-model R2>enable Password: (ciscoenpa55) R2#configure terminal R2(config)#aaa new-model R2 -> AAA -> Authentication -> Authen Command 1 R2(config)#aaa authentication login default group tacacs+ local R2 -> Console Line -> AAA Method List Name R2(config-line)#login authentication default R2(config-line)#exit R2 -> TACACS Client -> TACAS Server Hosts -> 0 tacacs-server host 192.168.2.2 R2 -> TACACS Client -> TACAS server key R2(config)#tacacs-server key tacacspa55 R2 -> User Names -> Username R2(config)#username Admin2 secret admin2pa55 Router R3 R3 -> AAA -> New-model R3>enable Password: (ciscoenpa55) R3#configure terminal R3(config)#aaa new-model R3 -> AAA -> Authentication -> Authen Command 1 R3(config)#aaa authentication login default group radius local R3 -> Console Line -> AAA Method List Name R3(config-line)#login authentication default R3(config-line)#exit R3 -> RADIUS Client -> RADIUS Server Hosts -> 0 R3(config)#radius-server host 192.168.3.2 R3 -> RADIUS Client -> RADIUS server key R3(config)# radius-server key radiuspa55 R3 -> User Names -> Username R3(config)#username Admin3 secret admin3pa55 |
| 5.4.1.2 Packet Tracer - Skills Integration Challenge.pka | R1(config)#interface G0/0 R1(config-if)#ip address 172.17.25.2 255.255.255.252 R1(config-if)#no shutdown R1(config-if)#exit R1 -> Ports -> GigabitEthernet0/1.10 -> 802.1Q -> VLAN ID R1(config-if)#interface G0/1.10 R1(config-subif)#encapsulation dot1Q 10 R1 -> Ports -> GigabitEthernet0/1.10 -> 802.1Q -> IP Address & Subnet Mask R1(config-subif)#ip address 172.17.10.1 255.255.255.0 R1(config-subif)#exit R1 -> Ports -> GigabitEthernet0/1.20 -> 802.1Q -> VLAN ID R1(config-if)#interface G0/1.20 R1(config-subif)#encapsulation dot1Q 20 R1 -> Ports -> GigabitEthernet0/1.20 -> 802.1Q -> IP Address & Subnet Mask R1(config-subif)#ip address 172.17.20.1 255.255.255.0 R1(config-subif)#exit R1 -> Ports -> GigabitEthernet0/1.30 -> 802.1Q -> VLAN ID R1(config-if)#interface G0/1.30 R1(config-subif)#encapsulation dot1Q 30 R1 -> Ports -> GigabitEthernet0/1.30 -> 802.1Q -> IP Address & Subnet Mask R1(config-subif)#ip address 172.17.30.1 255.255.255.0 R1(config-subif)#exit R1 -> Ports -> GigabitEthernet0/1.88 -> 802.1Q -> VLAN ID R1(config-if)#interface G0/1.88 R1(config-subif)#encapsulation dot1Q 88 R1 -> Ports -> GigabitEthernet0/1.88 -> 802.1Q -> IP Address & Subnet Mask R1(config-subif)#ip address 172.17.88.1 255.255.255.0 R1(config-subif)#exit R1 -> Ports -> GigabitEthernet0/1.99 -> 802.1Q -> VLAN ID R1(config-if)#interface G0/1.99 R1(config-subif)#encapsulation dot1Q 99 R1 -> Ports -> GigabitEthernet0/1.99 -> 802.1Q -> IP Address & Subnet Mask R1(config-subif)#ip address 172.17.99.1 255.255.255.0 R1(config-subif)#exit R1 -> Ports -> GigabitEthernet0/1 -> Port Status R1(config-subif)#interface G0/1 R1(config-if)#no shutdown Switch S1 S1 -> Default Gateway S1>enable S1#configure terminal S1(config)#ip default-gateway 172.17.99.1 S1 -> Ports -> FastEthernet0/10 -> Access VLAN S1(config)#interface range F0/6-10 S1(config-if-range)#switchport mode access S1(config-if-range)#switchport access vlan 30 S1(config-if-range)#exit S1 -> Ports -> FastEthernet0/11 -> Access VLAN S1(config)#interface range F0/11-17 S1(config-if-range)#switchport mode access S1(config-if-range)#switchport access vlan 10 S1(config-if-range)#exit S1 -> Ports -> FastEthernet0/18 -> Access VLAN S1(config)#interface range F0/18-24 S1(config-if-range)#switchport mode access S1(config-if-range)#switchport access vlan 20 S1(config-if-range)#exit S1 -> Vlan99 -> Post Status S1(config)#interface vlan 99 S1 -> Vlan99 -> IP Address & Subnet Mask S1(config-if)#ip address 172.17.99.10 255.255.255.0 S1(config-if)#exit S1 -> VLANS -> VLAN 10 -> VLAN Name S1(config)#vlan 10 S1(config-vlan)#name Faculty/Staff S1 -> VLANS -> VLAN 20 -> VLAN Name S1(config)#vlan 20 S1(config-vlan)#name Students S1 -> VLANS -> VLAN 30 -> VLAN Name S1(config)#vlan 30 S1(config-vlan)#name Guest(Default) S1(config-vlan)#exit S1 -> VLANS -> VLAN 88 -> VLAN Name S1(config)#vlan 88 S1(config-vlan)#name Native S1(config-vlan)#exit S1 -> VLANS -> VLAN 99 -> VLAN Name S1(config)#vlan 99 S1(config-vlan)#name Management S1(config-vlan)#exit S1 -> Ports -> GigabitEthernet0/1 S1(config)#interface G0/1 S1(config-if)#switchport trunk native vlan 88 S1(config-if)#exit Additional Requirements to shutdown all not assigned ports S1(config)#ip default-gateway 172.17.99.1 S1(config)#interface range F0/1-5, G0/2 S1(config-if-range)#shutdown S1(config-if-range)#exit |
| 6.6.1.2 Packet Tracer - Skills Integration Challenge.pka | HQ -> Routes -> Static Routes -> Route0 HQ>enable HQ# configure terminal HQ(config)#ip route 172.31.10.0 255.255.255.0 S0/0/0 HQ -> Routes -> Static Routes -> Route1 HQ(config)#ip route 172.31.20.0 255.255.255.0 S0/0/0 HQ -> Routes -> Static Routes -> Route2 HQ(config)#ip route 172.31.30.0 255.255.255.0 S0/0/0 HQ -> Routes -> Static Routes -> Route3 HQ(config)#ip route 172.31.88.0 255.255.255.0 S0/0/0 HQ -> Routes -> Static Routes -> Route4 HQ(config)#ip route 209.165.200.0 255.255.255.224 Serial0/1/0 HQ -> Routes -> Static Routes -> Route5 HQ(config)#ip route 209.165.200.0 255.255.255.224 Serial0/1/1 10 ISP -> Routes -> Static Routes ->Route1 ISP>enable ISP#configure terminal ISP(config)# ISP(config)#ip route 172.31.0.0 255.255.128.0 Serial0/1/1 ISP -> Routes -> Static Routes ->Route2 ISP(config)#ip route 172.31.0.0 255.255.128.0 Serial0/1/0 25 R1 -> Ports -> GigabitEthernet0/0 -> Port Status R1>enable R1#configure terminal R1(config-if)#no shutdown R1(config-if)#exit R1 -> Ports -> GigabitEthernet0/0.10 -> 802.1Q -> VLAN ID R1(config-if)#interface G0/0.10 R1(config-subif)#encapsulation dot1Q 10 R1 -> Ports -> GigabitEthernet0/0.10 -> 802.1Q -> IP Address & Subnet Mask R1(config-subif)#ip address 172.31.10.1 255.255.255.0 R1(config-subif)#exit R1 -> Ports -> GigabitEthernet0/0.20 -> 802.1Q -> VLAN ID R1(config-if)#interface G0/0.20 R1(config-subif)#encapsulation dot1Q 20 R1 -> Ports -> GigabitEthernet0/0.20 -> 802.1Q -> IP Address & Subnet Mask R1(config-subif)#ip address 172.31.20.1 255.255.255.0 R1(config-subif)#exit R1 -> Ports -> GigabitEthernet0/0.30 -> 802.1Q -> VLAN ID R1(config-if)#interface G0/0.30 R1(config-subif)#encapsulation dot1Q 30 R1 -> Ports -> GigabitEthernet0/0.30 -> 802.1Q -> IP Address & Subnet Mask R1(config-subif)#ip address 172.31.30.1 255.255.255.0 R1(config-subif)#exit R1 -> Ports -> GigabitEthernet0/0.88 -> 802.1Q -> VLAN ID R1(config-if)#interface G0/0.88 R1(config-subif)#encapsulation dot1Q 88 R1 -> Ports -> GigabitEthernet0/0.88 -> 802.1Q -> IP Address & Subnet Mask R1(config-subif)#ip address 172.31.88.1 255.255.255.0 R1(config-subif)#exit R1 -> Ports -> GigabitEthernet0/0.99 -> 802.1Q -> VLAN ID R1(config-if)#interface G0/0.99 R1(config-subif)# encapsulation dot1Q 99 native R1 -> Ports -> GigabitEthernet0/0.99 -> 802.1Q -> IP Address & Subnet Mask R1(config-subif)#ip address 172.31.99.1 255.255.255.0 R1(config-subif)#exit R1 -> Routes -> Static Routes -> Route0 R1(config)#ip route 0.0.0.0 0.0.0.0 Serial0/0/0 R1 -> Ports -> GigabitEthernet0/0 -> Port Status R1(config-subif)#interface G0/0 R1(config-if)#no shutdown R1(config-if)#exit S1 -> Ports -> GigabitEthernet0/1 -> Native VLAN S1>enable S1#configure terminal S1(config)#interface G0/1 S1(config-if)#switchport mode trunk S1(config-if)#switchport trunk native vlan 99 |
| 8.4.1.2_Packet_Tracer_-_Skills_Integration_Challenge[1](1).pka | Open ManagerA PC -> Desktop -> IP Configuration ManagerA -> Default Gateway Default Gateway: 172.16.10.1 ManagerA -> Default Gateway IPv6 IPv6 Gateway: FE80::1 ManagerA -> DNS Server IP IPv6 Gateway: 172.16.10.66 (IP Address of Website.pka) ManagerA -> DNS Server IPv6 IPv6 DNS Server: 2001:DB8:CAFE:2::2 (IPv6 Address of Website.pka) ManagerA -> Ports -> FastEthernet0 -> IP Address IP Address: 172.16.10.3 ManagerA -> Ports -> FastEthernet0 -> Subnet Mask Subnet Mask: 255.255.255.192 ManagerA -> Ports -> FastEthernet0 -> IPv6 Address -> IP Address IPv6 Address: 2001:DB8:CAFE:1::3 ManagerA -> Ports -> FastEthernet0 -> IPv6 Address -> Prefix Length / 64 Open R1 router -> CLI R1>enable R1#configure terminal R1 > Ports > GigabitEthernet0/0 > IP Address & Subnet Mask R1(config)#interface g0/0 R1(config-if)#ip address 172.16.10.1 255.255.255.192 R1 > Ports > GigabitEthernet0/0 > IPv6 Address > IP Address & Prefix R1(config-if)#ipv6 address 2001:DB8:CAFE:1::1/64 R1 > Ports > GigabitEthernet0/0 > Link Local R1(config-if)#ipv6 address FE80::1 link-local R1 > Ports > GigabitEthernet0/0 > Port Status R1(config-if)#no shutdown R1(config-if)#exit R1 > Ports > GigabitEthernet0/1 > IP Address & Subnet Mask R1(config)#interface g0/1 R1(config-if)#ip address 172.16.10.65 255.255.255.192 R1 > Ports > GigabitEthernet0/1 > IPv6 Address > IP Address & Prefix R1(config-if)#ipv6 address 2001:DB8:CAFE:2::1/64 R1 > Ports > GigabitEthernet0/0 > Link Local R1(config-if)#ipv6 address FE80::1 link-local R1 > Ports > GigabitEthernet0/0 > Port Status R1(config-if)#no shutdown R1(config-if)#exit Open S1 Switch -> CLI S1>enable S1#configure terminal S1 > Ports > Vlan1 > Default Gateway S1(config)#interface vlan 1 S1(config)#ip default-gateway 172.16.10.1 S1 > Ports > Vlan1 > IP Address & Subnet Mask S1(config-if)#ip address 172.16.10.62 255.255.255.192 S1 > Ports > Vlan1 > Port Status S1(config)#no shutdown Verify: Open ManagerA PC -> Desktop -> Command Prompt C:\>ping 172.16.10.2 C:\>ping 172.16.10.67 C:\>ping 172.16.10.66 C:\>ping 2001:DB8:CAFE:1::2 C:\>ping 2001:DB8:CAFE:2::3 C:\>ping 2001:DB8:CAFE:2::2 Open ManagerA PC -> Desktop -> Web Browser http://Accounting.pka http://Website.pka |
| 8.7.1.4_Packet_Tracer_-_Configure_and_Verify_a_Site-to-Site_IPsec_VPN_Using_CLI[1].pka | User Access Verification Password: (ciscoconpa55) R1>enable Password: (ciscoenpa55) R1#configure terminal R1 -> AGL -> 100 R1(config)#access-list 110 permit ip 192.168.1.0 0.0.0.255 192.168.3.0 0.0.0.255 R1 -> IKE -> Crypto IpSec Transform Sets -> Set VPN-SET -> Name R1(config-isakmp)#crypto ipsec transform-set VPN-SET esp-3des esp-sha-hmac R1 -> IKE -> Crypto ISAKMP Key Address Pairs -> vpnpa55 R1(config)#crypto isakmp key vpnpa55 address 10.2.2.2 R1 -> IKE -> Crypto ISAKMP Policy -> Authentication type R1(config)#crypto isakmp policy 10 R1(config-isakmp)#authentication pre-share R1 -> IKE -> Crypto ISAKMP Policy -> Encryption R1(config-isakmp)#encryption aes R1 -> IKE -> Crypto ISAKMP Policy -> Group R1(config-isakmp)#group 2 --- (Create the crypto map VPN-MAP) R1(config)#crypto map VPN-MAP 10 ipsec-isakmp R1(config-crypto-map)#description VPN connection to R3 R1(config-crypto-map)#set peer 10.2.2.2 R1(config-crypto-map)#set transform-set VPN-SET R1(config-crypto-map)#match address 110 R1(config-crypto-map)#exit R1(config)#interface S0/0/0 R1(config-if)#crypto map VPN-MAP R1(config-if)#exit --- Do the same for R3 Do next changes: a. R3(config)#access-list 110 permit ip 192.168.3.0 0.0.0.255 192.168.1.0 0.0.0.255 b. R3(config)#crypto isakmp key vpnpa55 address 10.1.1.2 |
| 9.3.2.10 Configuring Extended ACLs Scenario 1.pka | R1 -> ACL -> 100 R1>enable R1#configure terminal R1(config)#access-list 100 permit tcp 172.22.34.64 0.0.0.31 host 172.22.34.62 eq ftp R1(config)#access-list 100 permit icmp 172.22.34.64 0.0.0.31 host 172.22.34.62 R1 -> ACL -> HTTP_ONLY R1(config)#ip access-list extended HTTP_ONLY R1(config-ext-nacl)#permit tcp 172.22.34.96 0.0.0.15 host 172.22.34.62 eq www R1(config-ext-nacl)#permit icmp 172.22.34.96 0.0.0.15 host 172.22.34.62 R1(config-ext-nacl)#exit R1 -> GigabitEthernet0/0 -> Access-group in R1(config)#interface range G0/0 R1(config-if)#ip access-group 100 in R1(config-if)#exit R1 -> GigabitEthernet0/1 -> Access-group in R1(config)#interface G0/1 R1(config-if)#ip access-group HTTP_ONLY in R1(config-if)#exit PC2 -> From Browser: http://172.22.34.62 (OK) PC2 -> Command Prompt: C:\>ping 172.22.34.62. (OK) PC2 -> Command Prompt: C:\>ftp 172.22.34.62 Trying to connect...172.22.34.62 %Error opening ftp://172.22.34.62/ (Timed out) PC1 -> From Browser: http://172.22.34.62 -> Request Timeout PC1 -> Command Prompt: C:\>ping 172.22.34.62. (OK) PC1 -> Command Prompt: C:\>ftp 172.22.34.62 Trying to connect...172.22.34.62 Connected to 172.22.34.62 220- Welcome to PT Ftp server Username: cisco Password: cisco |
| 9.4.1.2_Packet_Tracer_-_Skills_Integration_Challenge[1](1).pka | Open Branch-A Router Branch-A -> Console Line -> Login & Password Router>enable Router# config terminal Router(config)#line console 0 Router(config-line)#password cisco Router(config-line)#login Router(config-line)#exit Branch-A -> Enable Secret Router(config)#enable secret class Branch-A -> Host Name Router(config)# hostname Branch-A Branch-A -> Ports -> Gig0/0 -> IP Address & Mask Branch-A(config-if)#ip address 172.20.16.1 255.255.254.0 Branch-A -> Ports -> Gig0/0 -> Port Status Branch-A(config-if)#no shutdown Branch-A -> Ports -> Gig0/1 -> IP Address & Mask Branch-A(config-if)#interface G0/1 Branch-A(config-if)#ip address 172.20.18.1 255.255.255.0 Branch-A -> Ports -> Gig0/1 -> Port Status Branch-A(config-if)#no shutdown Branch-A -> Ports -> Gig0/2 -> IP Address & Mask Branch-A(config-if)#interface G0/2 Branch-A(config-if)#ip address 172.20.31.254 255.255.255.252 Branch-A -> Ports -> Gig0/2 -> Port Status Branch-A(config-if)#no shutdown Branch-A(config-if)#exit Branch-A -> Service Password Encryption Branch-A(config)#service password-encryption Branch-A -> VTY Lines -> VTY Line 0 -> Password Branch-A(config)#line vty 0 15 Branch-A(config-line)#password cisco Branch-A(config-line)#exit Open Branch-B Router Branch-B -> Console Line -> Login & Password Router>enable Router# config terminal Router(config)#line console 0 Router(config-line)#password cisco Router(config-line)#login Router(config-line)#exit Branch-B -> Enable Secret Router(config)#enable secret class Branch-B -> Host Name Router(config)# hostname Branch-B Branch-B -> Ports -> G0/0 -> IPv6 Addresses -> 2001:DB8:FADE:00FF::1 Branch-B(config)#interface G0/0 Branch-B(config-if)#ipv6 address 2001:DB8:FADE:00FF::1/64 Branch-B -> Ports -> G0/0 -> IPv6 Addresses -> Link Local Branch-B(config-if)#ipv6 address FE80::B link-local Branch-B -> Ports -> G0/0 -> IPv6 Addresses -> Port Status Branch-B(config-if)#no shutdown Branch-B(config-if)#exit Branch-B -> Ports -> G0/1 -> IPv6 Addresses -> 2001:DB8:FADE:100::2 Branch-B(config)#interface G0/1 Branch-B(config-if)#ipv6 address 2001:DB8:FADE:100::1/64 Branch-B -> Ports -> G0/1 -> IPv6 Addresses -> Link Local Branch-B(config-if)#ipv6 address FE80::B link-local Branch-B -> Ports -> G0/1 -> IPv6 Addresses -> Port Status Branch-B(config-if)#no shutdown Branch-B(config-if)#exit Branch-B -> Ports -> G0/2 -> IPv6 Addresses -> 2001:DB8:FFFF:FFFF::2 Branch-B(config)#interface G0/2 Branch-B(config-if)#ipv6 address 2001:DB8:FFFF:FFFF::2/64 Branch-B -> Ports -> G0/2 -> IPv6 Addresses -> Link Local Branch-B(config-if)#ipv6 address FE80::B link-local Branch-B -> Ports -> G0/2 -> IPv6 Addresses -> Port Status Branch-B(config-if)#no shutdown Branch-B(config-if)#exit Branch-B -> Service Password Encryption Branch-B(config)#service password-encryption Branch-B -> VTY Lines -> VTY Line 0 -> Password Branch-B(config)#line vty 0 15 Branch-B(config-line)#password cisco Branch-B(config-line)#exit PC-A1 -> (all) Open PC-A1 -> Desktop -> IP Configuration IP Address: 172.20.17.254 Subnet Mask: 255.255.254.0 Default Gateway: 172.20.16.1 DNS Server: 172.20.32.10 PC-A2 -> (all) Open PC-A1 -> Desktop -> IP Configuration IP Address: 172.20.18.254 Subnet Mask: 255.255.255.0 Default Gateway: 172.20.18.1 DNS Server: 172.20.32.10 PC-B1 -> (all) Open PC-A1 -> Desktop -> IP Configuration IPv6 Address: 2001:DB8:FADE:00FF::10 / 64 IPv6 Gateway: FE80::B IPv6 DNS Server: 2001:DB8:FADE:1000::10 PC-B1 -> (all) Open PC-A1 -> Desktop -> IP Configuration IPv6 Address: 2001:DB8:FADE:0100::10 / 64 IPv6 Gateway: FE80::B IPv6 DNS Server: 2001:DB8:FADE:1000::10 |
| 9.4.1.5_Packet_Tracer_-_Configuring_ASA_Basic_Settings_and_Firewall_Using_CLI[1].pka | Click on 5505 ASA Firewall -> CLI ciscoasa>enable Password: ENTER (Verify: ciscoasa#show file system) (Verify: ciscoasa#show flash) (Verify: ciscoasa#show disk) (Verify: ciscoasa#show interface ip brief) (Verify: ciscoasa#show ip address) (Verify: ciscoasa#show switch vlan) (Verify: ciscoasa#show route) (Verify: ciscoasa#show nat) (Verify: ciscoasa#show running-config) ASA -> AccessGroup List ciscoasa(config)#access-group OUTSIDE-DMZ in interface outside ASA -> OUTSIDE-DMZ ciscoasa(config)#access-list OUTSIDE-DMZ permit icmp any host 192.168.2.3 ciscoasa(config)#access-list OUTSIDE-DMZ permit tcp any host 192.168.2.3 eq 80 ASA -> Asa Object List -> dmz-server -> Address ciscoasa(config-if)#object network dmz-server ciscoasa(config-network-object)#host 192.168.2.3 ASA -> Asa Object List -> dmz-server -> Nat ASA -> ?asl_dmzserver? ciscoasa(config-network-object)#nat (dmz,outside) static 209.165.200.227 ciscoasa(config-network-object)#exit ASA -> Asa Object List -> inside-net-> Address ciscoasa(config)#object network inside-net ciscoasa(config-network-object)#subnet 192.168.1.0 255.255.255.0 ASA -> DHCP Server List -> DHCP Server (inside) -> Pools -> Pool serverPool -> Start IP Address & Subnet mask ciscoasa#dhcpd address 192.168.1.5-192.168.1.36 inside ciscoasa(config)#dhcpd dns 209.165.201.2 interface inside ASA -> DHCP Server List -> DHCP Server (inside) -> DHCP Enable ciscoasa(config)#dhcpd enable inside ASA -> DHCP Server List -> DHCP Server (inside) -> DHCP Auto_config ciscoasa(config)#dhcpd auto_config inside ASA > ACL -> ?acl_inside-net? ASA > Asa Object List -> inside-net-> Nat ciscoasa(config-network-object)#nat (inside,outside) dynamic interface ciscoasa(config-network-object)#end ASA -> Class Maps -> Class Map List -> Inspection default -> Map Type & Statements ciscoasa(config)#class-map inspection_default ciscoasa(config-cmap)#match default-inspection-traffic ciscoasa(config-cmap)#exit ASA -> Host Name ciscoasa#configure terminal ciscoasa(config)#hostname CCNAS-ASA ASA -> IP Domain Name CCNAS-ASA(config)#domain-name ccnasecurity.com ASA -> Management Access List -> Access Setting (1) CCNAS-ASA(config)#telnet 192.168.1.0 255.255.255.0 inside ASA -> Management Access List -> Access Setting (2) CCNAS-ASA(config)#ssh 192.168.1.0 255.255.255.0 inside ASA -> Management Access List -> Access Setting (3) CCNAS-ASA(config)#ssh 172.16.3.3 255.255.255.255 outside ASA -> Management Access List -> Ssh Timeout CCNAS-ASA(config)#ssh timeout 10 ASA -> Management Access List -> Telnet Timeout CCNAS-ASA(config)#telnet timeout 10 ASA -> Policy Maps -> Policy Map List -> Policy Map global_policy CCNAS-ASA(config)#policy-map global_policy CCNAS-ASA(config-pmap)#class inspection_default CCNAS-ASA(config-pmap-c)#inspect icmp CCNAS-ASA(config-pmap-c)#exit CCNAS-ASA(config)#service-policy global_policy global ASA -> Ports -> Vlan1 -> IP Address & Subnet Mask CCNAS-ASA(config)#interface vlan 1 CCNAS-ASA(config-if)#security-level 100 CCNAS-ASA(config-if)#ip address 192.168.1.1 255.255.255.0 ASA -> VLANS > VLAN 1 CCNAS-ASA(config-if)#nameif inside CCNAS-ASA(config-if)#exit ASA -> Ports -> Vlan2 -> IP Address & Subnet Mask CCNAS-ASA(config)#interface vlan 2 CCNAS-ASA(config-if)#security-level 0 CCNAS-ASA(config-if)#ip address 209.165.200.226 255.255.255.248 ASA -> VLANS > VLAN 2 CCNAS-ASA(config-if)#nameif outside CCNAS-ASA(config-if)#exit ASA -> Ports -> Vlan3 -> IP Address & Subnet Mask CCNAS-ASA(config)#interface vlan 3 CCNAS-ASA(config-if)#ip address 192.168.2.1 255.255.255.0 ASA -> Ports -> Vlan3 -> No Forward CCNAS-ASA(config)#no forward interface vlan 1 ASA -> Ports -> Vlan3 -> Security Level CCNAS-ASA(config)#security-level 70 ASA -> VLANS > VLAN 3 ASA -> Ports -> Vlan3 -> NameIf CCNAS-ASA(config-if)#nameif dmz CCNAS-ASA(config-if)#exit ASA -> Routers -> Static Routes -> Route0 CCNAS-ASA(config)#route outside 0.0.0.0 0.0.0.0 209.165.200.225 ASA -> PC-B -> Ports -> FastEthernet0 -> DHCP client enable Click on PC-PT / PC-B a. Desktop -> IP Configuration b. Switch from Static to DHCP IP configuration No points: CCNAS-ASA(config)#enable password ciscoenpa55 CCNAS-ASA(config)#clock set 14:30:00 Dec 7 2019 CCNAS-ASA#username admin password cisco123 CCNAS-ASA(config)#aaa authentication ssh console LOCAL CCNAS-ASA(config)#aaa authentication telnet console LOCAL CCNAS-ASA(config-if)#interface Ethernet0/2 CCNAS-ASA(config-if)#switchport access vlan 3 |
| 10.3.1.2 Packet Tracer - Skills Integration Challenge.pka | Click on R1 Router -> CLI R1>enable R1#show running-config R1 > DHCP Server -> Excluded Addresses -> (deprecated) R1(config)#ip dhcp excluded-address 172.31.10.1 172.31.10.10 R1(config)#ip dhcp excluded-address 172.31.20.1 172.31.20.10 R1(config)#ip dhcp excluded-address 172.31.30.1 172.31.30.10 R1(config)#ip dhcp excluded-address 172.31.40.1 172.31.40.10 R1 > Pools -> Pool VLAN_10 -> Name R1(config)#ip dhcp pool VLAN_10 R1 > Pools -> Pool VLAN_10 -> Default Gateway R1(dhcp-config)#default-router 172.31.10.1 R1 > Pools -> Pool VLAN_10 -> DNS server IP R1(dhcp-config)#dns-server 209.165.201.14 R1 > Pools -> Pool VLAN_10 -> Start IP address & Subnet mask R1(dhcp-config)#network 172.31.10.0 255.255.255.224 R1 > Pools -> Pool VLAN_20 -> Name R1(config)#ip dhcp pool VLAN_20 R1 > Pools -> Pool VLAN_20 -> Default Gateway R1(dhcp-config)#default-router 172.31.20.1 R1 > Pools -> Pool VLAN_20 -> DNS server IP R1(dhcp-config)#dns-server 209.165.201.14 R1 > Pools -> Pool VLAN_10 -> Start IP address & Subnet mask R1(dhcp-config)#network 172.31.20.0 255.255.255.240 R1 > Pools -> Pool VLAN_30 -> Name R1(config)#ip dhcp pool VLAN_30 R1 > Pools -> Pool VLAN_30 -> Default Gateway R1(dhcp-config)#default-router 172.31.30.1 R1 > Pools -> Pool VLAN_30 -> DNS server IP R1(dhcp-config)#dns-server 209.165.201.14 R1 > Pools -> Pool VLAN_30 -> Start IP address & Subnet mask R1(dhcp-config)#network 172.31.30.0 255.255.255.128 R1 > Pools -> Pool VLAN_40 -> Name R1(config)#ip dhcp pool VLAN_40 R1 > Pools -> Pool VLAN_40 -> Default Gateway R1(dhcp-config)#default-router 172.31.40.1 R1 > Pools -> Pool VLAN_40 -> DNS server IP R1(dhcp-config)#dns-server 209.165.201.14 R1 > Pools -> Pool VLAN_40 -> Start IP address & Subnet mask R1(dhcp-config)#network 172.31.40.0 255.255.255.192 R1(dhcp-config)#exit PC(x) -> Default Gateway, DNS Server IP & Ports Open PC -> Desktop -> IP Configuration Switch to DHCP (IP Configuration) R1 > Ports -> GigabitEthernet0/0.10 -> 802.1Q -> VLAN ID R1#configure terminal R1(config)#interface g0/0.10 R1(config-subif)#encapsulation dot1Q 10 R1 > Ports -> GigabitEthernet0/0.10 -> IP Address & Subnet Mask R1(config-subif)#ip address 172.31.10.1 255.255.255.224 R1 > Ports -> GigabitEthernet0/0.20 -> 802.1Q -> VLAN ID R1(config)#interface g0/0.20 R1(config-subif)#encapsulation dot1Q 20 R1 > Ports -> GigabitEthernet0/0.20 -> IP Address & Subnet Mask R1(config-subif)#ip address 172.31.20.1 255.255.255.240 R1 > Ports -> GigabitEthernet0/0.30 -> 802.1Q -> VLAN ID R1(config)#interface g0/0.30 R1(config-subif)#encapsulation dot1Q 30 R1 > Ports -> GigabitEthernet0/0.30 -> IP Address & Subnet Mask R1(config-subif)#ip address 172.31.30.1 255.255.255.128 R1 > Ports -> GigabitEthernet0/0.40 -> 802.1Q -> VLAN ID R1(config)#interface g0/0.40 R1(config-subif)#encapsulation dot1Q 40 R1 > Ports -> GigabitEthernet0/0.40 -> IP Address & Subnet Mask R1(config-subif)#ip address 172.31.40.1 255.255.255.192 R1(config-subif)#exit R1 > Ports -> GigabitEthernet0/0 -> Port Status R1(config)#interface g0/0 R1(config-if)#no shutdown R1(config-if)#exit R1 > Ports -> GigabitEthernet0/1 -> DHCP client enable R1(config)#interface g0/1 R1(config-if)#ip address dhcp R1 > Ports -> GigabitEthernet0/1 -> Port Status R1(config)#no shutdown R1 > Ports -> GigabitEthernet0/1 -> IP Address & Subnet Mask Just wait one minute when DHCP address assigned to the interface… %DHCP-6-ADDRESS_ASSIGN: Interface GigabitEthernet0/1 assigned DHCP address 209.165.200.227, mask 255.255.255.224, hostname R1 Click on S2 Switch -> CLI ENTER S2>enable S2#configure terminal S2 > Ports -> FastEthernet0/5 -> Access & VLAN S2(config-if-range)#interface range fa0/5-9 S2(config-if-range)#switchport access vlan 10 S2 > Ports -> FastEthernet0/12 -> Access VLAN S2(config-if-range)#interface range fa0/10-14 S2(config-if-range)#switchport access vlan 20 S2 > Ports -> FastEthernet0/15 -> Access VLAN S2(config-if-range)#interface range fa0/15-19 S2(config-if-range)#switchport access vlan 30 S2 > Ports -> FastEthernet0/20 -> Access VLAN S2(config)#interface range fa0/20-24 S2(config-if-range)#switchport access vlan 40 S2(config-if-range)#exit S2 > Ports -> FastEthernet0/1 (0/2,0/3,0/4) ->Dynamic Mode S2(config)#interface range fa0/1-4 S2(config-if-range)#switchport mode trunk S2(config-if-range)#exit S2 > Ports -> FastEthernet0/5-24 (All non trunk ports) -> Dynamic Mode S2(config)#interface range fa0/5-24, g0/1-2 S2(config-if-range)#switchport mode access S2(config-if-range)#exit S2 > VLANS -> VLAN (x) -> VLAN Name S2(config)#vlan 10 S2(config-vlan)#name Sales S2(config-vlan)#vlan 20 S2(config-vlan)#name Production S2(config-vlan)#vlan 30 S2(config-vlan)#name Marketing S2(config-vlan)#vlan 40 S2(config-vlan)#name HR |
| 11.4.1.2 Packet Tracer - Skills Integration Challenge.pka | Open Admin Router -> CLI Admin# enable Admin#configure terminal Admin -> ACL -> 1 Admin(config)#access-list 1 permit 10.10.10.0 0.0.0.255 Admin -> DHCP Server -> Pools -> Pool LAN -> Default Gateway Admin(config)#ip dhcp pool LAN Admin(dhcp-config)#default-router 10.10.10.193 Admin -> DHCP Server -> Pools -> Pool LAN -> Start IP address & Subnet Admin(dhcp-config)#network 10.10.10.192 255.255.255.192 Admin(dhcp-config)#exit Admin -> NAT -> Pools -> Pool Name 1 Admin(config)#ip nat pool INTERNET 198.133.219.128 198.133.219.129 netmask 255.255.255.252 Admin -> NAT -> Inside Source List -> NAT Source Setting 1 Admin(config)#ip nat inside source list 1 pool INTERNET overload Admin -> NAT -> Inside Source Static -> NAT Source Setting 1 ip nat inside source static 10.10.10.162 192.133.219.130 (not working) Admin -> OSPF -> Process ID 1 -> Networks -> Router0 Admin(config)#router ospf 1 Admin(config-router)#network 10.10.10.0 0.0.0.255 area 0 Admin -> OSPF -> Process ID 1 -> Passive Interface -> GigabitEthernet0/0 Admin(config-router)#passive-interface g0/0.15 Admin(config-router)#passive-interface g0/0.30 Admin(config-router)#passive-interface g0/0.45 Admin(config-router)#passive-interface g0/0.60 Admin(config-router)#passive-interface s0/1/0 Admin(config-router)#passive-interface g0/0 Admin -> OSPF -> Process ID 1 -> Router ID Admin(config-router)#router-id 1.1.1.1 Admin(config-router)#exit Admin -> Ports -> GigabitEthernet0/0.15 -> 802.1Q -> VLAN ID Admin(config)#interface g0/0.15 Admin(config-subif)#encapsulation dot1Q 15 Admin -> Ports -> GigabitEthernet0/0.15 -> IP Address & Subnet Mask Admin(config-subif)#ip address 10.10.10.161 255.255.255.224 Admin -> Ports -> GigabitEthernet0/0.15 -> NAT Mode Admin(config-subif)#ip nat inside Admin -> Ports -> GigabitEthernet0/0.30 -> 802.1Q -> VLAN ID Admin(config)#interface g0/0.30 Admin(config-subif)#encapsulation dot1Q 30 Admin -> Ports -> GigabitEthernet0/0.30 -> IP Address & Subnet Mask Admin(config-subif)#ip address 10.10.10.192 255.255.255.192 Admin -> Ports -> GigabitEthernet0/0.30 -> NAT Mode Admin(config-subif)#ip nat inside Admin -> Ports -> GigabitEthernet0/0.45 -> 802.1Q -> VLAN ID Admin -> Ports -> GigabitEthernet0/0.45 -> 802.1Q -> Native VLAN Admin(config)#interface g0/0.45 Admin(config-subif)#encapsulation dot1Q 45 native Admin -> Ports -> GigabitEthernet0/0.45 -> IP Address & Subnet Mask Admin(config-subif)#ip address 10.10.10.129 255.255.255.240 Admin -> Ports -> GigabitEthernet0/0.60 -> 802.1Q -> VLAN ID Admin(config)#interface g0/0.60 Admin(config-subif)#encapsulation dot1Q 60 Admin -> Ports -> GigabitEthernet0/0.60 -> IP Address & Subnet Mask Admin(config-subif)#ip address 10.10.10.145 255.255.255.240 Admin(config-subif)#exit Admin -> Ports -> GigabitEthernet0/0 -> Port Status Admin(config-subif)#interface g0/0 Admin(config-if)#no shutdown Admin(config-if)#exit Admin -> Serial0/0/0 -> NAT Mode Admin(config)#interface s0/0/0 Admin(config-if)#ip nat inside Admin -> Serial0/0/1 -> NAT Mode Admin(config)#interface s0/0/1 Admin(config-if)#ip nat inside Admin -> Serial0/1/0 -> NAT Mode Admin(config)#interface s0/1/0 Admin(config-if)#ip nat outside Admin -> Routes -> Static Routes -> Route0 Admin(config)#ip route 0.0.0.0 0.0.0.0 s0/1/0 Open Admin-Sw Switch -> CLI Admin-Sw -> Default Gateway Admin-Sw(config)#ip default-gateway 10.10.10.145 Admin-Sw -> IP Domain Name Admin-Sw>enable Admin-Sw#configure terminal Admin-Sw(config)#ip domain-name cisco.com Admin-Sw -> Ports -> FastEthernet0/1 -> Access VLAN Admin-Sw(config)#interface range f0/1-10 Admin-Sw(config-if-range)#switch access vlan 30 Admin-Sw -> Ports -> FastEthernet0/1 -> Dynamic Mode Admin-Sw(config-if-range)#switch mode access Admin-Sw -> Ports -> FastEthernet0/1 -> Port Security -> Enabled Admin-Sw(config-if)#interface f0/1 Admin-Sw(config-if)#switchport port-security Admin-Sw -> Ports -> FastEthernet0/1 -> Port Security -> Maximum Static MACs Admin-Sw(config-if)#switchport port-security maximum 2 Admin-Sw -> Ports -> FastEthernet0/1 -> Port Security -> Port Security Violation Admin-Sw(config-if)#switchport port-security violation restrict Admin-Sw -> Ports -> FastEthernet0/1 -> Port Security -> Sticky Enabled Admin-Sw(config-if)#switchport port-security mac-address sticky Admin-Sw(config-if)#exit Admin-Sw -> Ports -> FastEthernet0/11 -> Access VLAN Admin-Sw(config)#interface range f0/11-20 Admin-Sw(config-if-range)#switch access vlan 15 Admin-Sw -> Ports -> FastEthernet0/11 -> Dynamic Mode Admin-Sw(config-if-range)#switch mode access Admin-Sw(config-if-range)#exit Admin-Sw -> Ports -> GigabitEthernet0/1 -> Dynamic Mode Admin-Sw(config)#interface g0/1 Admin-Sw(config-if)#switchport mode trunk Admin-Sw -> Ports -> GigabitEthernet0/1 -> Native VLAN Admin-Sw(config-if)#switchport trunk native vlan 45 Admin-Sw(config-if)#exit Admin-Sw -> Ports -> FastEthernet0/21 -> Port Status Admin-Sw -> Ports -> GigabitEthernet0/2 -> Port Status Admin-Sw(config)#interface range f0/2-10, f0/12-24, g0/2 Admin-Sw(config-if-range)#shutdown Admin-Sw(config-if-range)#exit Admin-Sw -> Ports -> Vlan60 -> Port Status Admin-Sw(config)#interface vlan 60 Admin-Sw -> Ports -> Vlan60 -> IP Address & Subnet Admin-Sw(config-if)#ip address 10.10.10.146 255.255.255.240 Admin-Sw(config-if)#no shutdown Admin-Sw(config-if)#exit Admin-Sw -> Service Password Encyption Admin-Sw(config)#service password-encryption Admin-Sw -> SSH Server -> SSH Authentication Retries Admin-Sw(config)#ip ssh authentication-retries 2 Admin-Sw -> SSH Server -> SSH Timeout Admin-Sw(config)#ip ssh time-out 60 Admin-Sw -> SSH Server -> SSH Version Admin-Sw(config)#ip ssh version 2 Admin-Sw -> User Names -> Username (no points) Admin-Sw(config)#username Admin password letmein Admin-Sw -> VLANS -> VLAN (x) Admin-Sw(config)#vlan 15 Admin-Sw(config-vlan)#name Servers Admin-Sw(config-vlan)#vlan 30 Admin-Sw(config-vlan)#name PCs Admin-Sw(config-vlan)#vlan 45 Admin-Sw(config-vlan)#name Native Admin-Sw(config-vlan)#vlan 60 Admin-Sw(config-vlan)#name Management Admin-Sw(config-vlan)#exit Admin-Sw -> VTY Lines -> VTY Line 0 -> Login Admin-Sw(config)#line vty 0 15 Admin-Sw(config-line)#login local Admin-Sw -> VTY Lines -> VTY Line 0 -> Transport Input Admin-Sw(config-line)#transport input ssh Open “IP support” PC-VLAN 30 Itsupport > Ports > FastEthernet0 > DHCP client enable & Subnet Mask Desktop → IP Configuration → DHCP No Points -Crypto key length of 1024 Admin-Sw(config)#crypto key generate rsa How many bits in the modulus [512]: 1024 Admin(config)#router ospf 1 Admin(config-router)#default-information originate Admin(config)#interface g0/0.45 Admin(config-subif)#ip nat inside Admin(config)#interface g0/0.60 Admin(config-subif)#ip nat inside Admin(config)#ip nat inside source static 10.10.10.162 192.133.219.130 |